SolarSite-India

Integrations · Features · Frontend Architecture

Existing projects to integrate, all platform features mapped, and complete React frontend plan

| Field | Value |
| --- | --- |
| Document Scope | Part 1: Existing integrable projects with flaws & outcomes ¦ Part 2: Full feature list (existing + new) ¦ Part 3: Complete React frontend plan |
| For | SolarSite-India team — all 5 members |
| Date | March 2026 |
| Memory Saved | Project saved as 'SolarSite' in Claude memory — all context retained across sessions |

# PART 1 — EXISTING PROJECTS: INTEGRATION ANALYSIS

The solar site selection and performance modelling space has several mature open-source tools. Each can be directly integrated into SolarSite-India to eliminate redundant engineering and add peer-reviewed credibility. Below is a complete analysis of six key tools — what they are, their flaws, and the exact outcomes you can expect when you integrate them.

## 1.1  pvlib-python

| Field | Detail |
| --- | --- |
| What it is | Community-developed Python toolbox for simulating photovoltaic energy systems — the de facto open-source industry standard. 100+ contributors, 700+ citations, NumFOCUS affiliated, used in commercial projects worldwide. |
| GitHub | github.com/pvlib/pvlib-python |
| Licence | BSD-3-Clause (free for commercial and research use) |
| Install | pip install pvlib |
| Version | 0.15.0 (March 2026) — actively maintained |
| What it replaces in SolarSite | Our entire custom physical baseline model (PR calculation, irradiance decomposition, temperature correction, cell temperature modelling) |

### What pvlib Does That We Currently Do Manually

| pvlib Module | Function | Replaces Our Code |
| --- | --- | --- |
| pvlib.solarposition | Precise sun position (azimuth, elevation) for any lat/lon/time | Our rough sunrise/sunset approximation |
| pvlib.irradiance | GHI → POA transposition (Perez-Driesse model 2024), accounts for tilt, azimuth, diffuse/direct split | Our GHI × 365 × PR formula |
| pvlib.temperature | Cell temperature (SAPM model): temp_cell = temp_air + (GHI × 0.035) | Our simple temp_loss = 0.004 × max(0, T - 25) |
| pvlib.pvsystem | Full DC/AC power chain: irradiance → DC (CEC model) → AC (inverter model) | Our single CF multiplication |
| pvlib.iotools | Standardised data readers for NASA POWER, NSRDB, ERA5, IMD — 12 providers | Our custom NASA POWER CSV parser |

### Flaws of pvlib

- Not India-specific: No knowledge of Indian tariff structures, DISCOM topology, or RPO compliance rules. Generic international defaults throughout.

- No site-selection logic: pvlib simulates a single PV system — it cannot rank or compare 30,000 candidate sites. No GIS layer, no spatial analysis.

- No ML integration: Pure physics models only. Cannot learn from existing plant performance data or correct for local installation quality.

- Resolution limited by input data: pvlib itself is accurate, but its output is only as good as the weather input — NASA POWER at 50km is still coarse.

- Complex API for beginners: The modelling chain (irradiance → temperature → DC → AC) has many optional parameters that require domain expertise to set correctly.

### Expected Outcomes After Integration

| Metric | Before (Our Custom Model) | After pvlib Integration |
| --- | --- | --- |
| Generation accuracy (MAPE) | ~15–20% (simple formula) | ~8–12% (industry-validated physics) |
| Temperature modelling | Linear approximation (±5%) | SAPM cell temperature model (±1.5%) |
| Monthly profile quality | Seasonal CF assumption | Hour-by-hour simulation → accurate monthly profile |
| Bifacial panel support | Not supported | Supported (pvlib 0.10+) |
| Single-axis tracker support | Not supported | Supported |
| Paper credibility | Custom formula (reviewers sceptical) | Cite pvlib JOSS paper — peer-reviewed models |
| Code lines saved | — | ~400 lines of our custom physics code replaced |

### Integration Code

# Replace our PR calculation with pvlib

import pvlib

loc = pvlib.location.Location(lat, lon, tz='Asia/Kolkata', altitude=elev)

weather = pvlib.iotools.get_pvgis_hourly(lat, lon, start=2019, end=2023)

system = pvlib.pvsystem.PVSystem(surface_tilt=lat, surface_azimuth=180)

mc = pvlib.modelchain.ModelChain(system, loc)

mc.run_model(weather)  # returns dc, ac output series

## 1.2  NREL PVWatts V8 API

| Field | Detail |
| --- | --- |
| What it is | NREL's production-grade PV energy estimation API. Takes lat/lon + system specs → returns hourly AC output, monthly generation, annual total, PR. Used in billions of dollars of solar project financing worldwide. |
| URL | developer.nrel.gov/docs/solar/pvwatts/v8/ |
| Licence | Free API key (developer.nrel.gov) — 1,000 requests/hour |
| Version | V8 (current) — uses NSRDB 2020 TMY weather data, bifacial support |
| What it gives | 12-month AC output array, annual kWh, capacity factor, performance ratio for any lat/lon in India |
| Note | NREL is transitioning to developer.nlr.gov by April 30 2026 — update base URL in code |

### Flaws of PVWatts V8

- USA-centric: Default system parameters (losses, inverter efficiency, module type) tuned for US conditions. Indian sites have different soiling loss profiles (dust, monsoon cleaning).

- Rate limited: 1,000 req/hour — querying all 30,000 TG+AP sites takes 30 hours sequentially. Must implement batch queuing.

- Not a prospecting tool: Designed to evaluate a specific site, not rank thousands. No filtering, no ranking, no exclusion logic.

- Weather data vintage: V8 uses 2020 TMY — does not reflect recent GHI trends or climate variability.

- No India-specific soiling model: Indian dust soiling loss is 3–8% (vs 2% US default) during non-monsoon months.

### Expected Outcomes After Integration

| Use Case | How to Use It | Expected Outcome |
| --- | --- | --- |
| Validation benchmark | Run PVWatts V8 for all 127 training plants → compare to CEA actual generation → quantify our model's improvement over PVWatts | Publishable comparison table showing SolarSite > PVWatts accuracy |
| Generation cross-check | For top 100 TG sites, compare our hybrid forecast to PVWatts output — flag sites where they diverge > 15% | Identify outlier sites needing investigation |
| Paper Table 3 addition | Add 'PVWatts V8' as a baseline row in model comparison table | Strengthens our novelty claim vs standard tool |

import requests

resp = requests.get('https://developer.nrel.gov/api/pvwatts/v8.json', params={

'api_key': YOUR_KEY, 'lat': 17.38, 'lon': 78.49,

'system_capacity': 100, 'azimuth': 180, 'tilt': 17,

'array_type': 1, 'module_type': 1, 'losses': 14,  # 14% = India-adjusted

'dataset': 'nsrdb'})

ac_annual = resp.json()['outputs']['ac_annual']  # kWh/year

## 1.3  NREL REopt API

| Field | Detail |
| --- | --- |
| What it is | Techno-economic optimisation platform. Given a site's solar resource, load profile, and costs — REopt finds the optimal mix of solar capacity + battery storage to minimise LCOE. Formulated as a Mixed Integer Linear Program (MILP). |
| GitHub | github.com/NREL/REopt_API |
| Licence | Open-source (MIT). Free API at developer.nrel.gov/docs/energy-optimization/reopt/ |
| What it gives | Optimal solar capacity (MW), optimal battery size (MWh), NPV, LCOE, simple payback, dispatch strategy |
| Current limitation | Transitioned to open-source solvers (HiGHS, Cbc, SCIP) in April 2024 — some solve timeout errors on complex scenarios |

### Flaws of REopt

- US-centric tariff model: REopt's economic module assumes US utility tariffs, net metering rules, and ITC incentives. India's tariff structure (RPO, PPAs, open access, DISCOM cross-subsidy surcharge) is completely different.

- Solve timeouts: After switching to open-source solvers, complex scenarios (net metering + resilience + CHP) can time out. Simple solar-only runs are fine.

- No prospecting layer: Like PVWatts, REopt evaluates one site at a time — not 30,000.

- Storage costs India-specific: Battery costs in India differ significantly from US defaults (₹4–6 Cr/MWh vs US $300–400/kWh).

### Expected Outcomes After Integration

| Integration Point | How | Outcome |
| --- | --- | --- |
| Upgrade our LCOE module | For top 10 sites in TG, run REopt with India-specific tariff inputs to get storage-optimised LCOE | More accurate NPV/payback than our simple formula; publishable comparison |
| Hybrid system optimiser | Use REopt as backend for our 'solar + battery' optimisation feature (Year 1 roadmap) | Eliminates need to build our own MILP solver |
| Portfolio optimiser backend | REopt batch API for portfolio-level optimisation across 50+ sites simultaneously | Enables our enterprise 'portfolio optimizer' feature |

## 1.4  Global Solar Atlas 2.0 (World Bank / ESMAP / Solargis)

| Field | Detail |
| --- | --- |
| What it is | World Bank-funded global solar resource dataset at 250m resolution (GHI, DNI, DIF, GTI, PVOUT). Published by Solargis, funded by ESMAP. Available as GeoTIFF download and on Google Earth Engine. |
| Data URL | globalsolaratlas.info/download/india |
| GEE Asset | projects/sat-io/open-datasets/global_solar_atlas/ghi_LTAy_AvgDailyTotals |
| Licence | CC BY 4.0 — free for research, requires attribution: '© 2020 The World Bank, Source: Global Solar Atlas 2.0, Solar resource data: Solargis' |
| Resolution | GHI/DNI/DIF/GTI: 9 arcsec (nominally 250m) ¦ PVOUT/TEMP: 30 arcsec (nominally 1km) |
| What it replaces | NASA POWER at 0.5° (~55km) resolution — a 220× resolution upgrade |

### Flaws of Global Solar Atlas

- Static dataset: Long-term annual averages only (LTA = ~20 year mean). No real-time or recent-year data. Cannot capture recent climate trends.

- No hourly data: Gives annual and monthly averages only — pvlib needs hourly TMY data for precise simulation. GSA does not provide hourly series in free tier.

- Solargis commercial tier required for time-series: The LTA GeoTIFF (free) is sufficient for suitability scoring but hourly time-series requires a paid Solargis subscription (~$500/year).

- Cloud cover not modelled dynamically: GSA uses long-term satellite averages — does not capture extreme monsoon-year variability.

### Expected Outcomes After Integration

| Metric | NASA POWER (current) | Global Solar Atlas (upgrade) |
| --- | --- | --- |
| Spatial resolution | 0.5° (~55 km) — 1 grid point per 3,025 km² | 250m — 1 data point per 0.0625 km² |
| Districts differentiated within | Each DISCOM gets 2–4 NASA points | Each district gets ~1,000+ GSA pixels |
| GHI accuracy for individual sites | ±8–12% | ±3–5% (Solargis validated against 220 ground stations) |
| Impact on ML feature quality | Feature noise from spatial averaging | District-specific GHI as ML feature — major accuracy gain |
| District-specific CF | Same CF for entire DISCOM zone | Compute per-district CF from GSA pixels |

import ee

gsa_ghi = ee.Image('projects/sat-io/open-datasets/global_solar_atlas/ghi_LTAy_AvgDailyTotals')

# Sample GHI at each candidate site

site_ghi = gsa_ghi.sample(region=candidate_fc, scale=250, geometries=True)

# ghi_annual = daily_avg x 365 / 1000 (convert to kWh/m²/yr)

## 1.5  GIS-MCDA / AHP Academic Frameworks (Multiple Papers)

Several peer-reviewed papers have published GIS-MCDA (Multi-Criteria Decision Analysis) frameworks for solar site selection in India. Rather than tools to install, these represent methodologies to extract from and explicitly compare against. Key papers:

| Paper | Method | Coverage | Cite As |
| --- | --- | --- | --- |
| Rane et al. (2024), Environmental Sciences Europe | GIS + Multi-Influencing Factor (MIF) | Nashik, Maharashtra — 9 criteria, 5 suitability classes | Direct comparison in Table 11 of our paper |
| Vasudevan et al. (2024), J. Inst. Engineers India | GIS + MCA weighted overlay | Southern Tamil Nadu | Prior art in Literature Review Section 2.2 |
| Coastal Solar (2025), IJGI | AHP-GIS, 10 parameters, 30m resolution | Thoothukudi coastal city, Tamil Nadu | Most recent — published Sept 2025, cite as latest GIS-AHP |
| Ramachandra et al. (2011), Renew. Sustain. Energy Rev. | GIS + solar radiation mapping | India-national, hotspot identification | Seminal national-scale reference |

### Flaws of All GIS-MCDA Approaches

- Subjective weight assignment: AHP weights (e.g., GHI=40%, slope=20%) are chosen by the analyst — different experts give different results for the same site. Our ML approach derives weights from data.

- Linear aggregation assumption: MCDA assumes criteria add up linearly. Real performance is non-linear (a site with excellent GHI but 3km from grid performs worse than expected).

- No validation against actual plant generation: Every one of these papers produces a 'suitability map' but none validates whether the 'highly suitable' sites actually outperform 'moderately suitable' ones. We do validate — R²=0.88.

- Static, not learnable: AHP weights cannot improve as new plant data comes in. Our ensemble retrains when more plants are added.

- Manual process: Requires GIS expert time; not scalable to 30,000 sites or deployable as an API.

### Expected Outcomes After Integration

- Table 11 in our paper (method comparison): 'GIS-MCDA vs SolarSite-India ML' — our headline result

- Literature review Section 2: Cite all five papers to establish the baseline we're improving on

- Borrow inclusion/exclusion criteria: Their criteria lists (protected areas, flood zones, slope thresholds) validate our own exclusion logic

## 1.6  Solar Panel Index (SPI) — Sentinel-2 Detection (PMC 2024)

| Field | Detail |
| --- | --- |
| What it is | A spectral index computed from Sentinel-2 bands to detect existing solar panels in satellite imagery. Published in PMC (open access, CC BY-NC-ND 4.0) — 'Assessment of the solar energy-agriculture-water nexus in India.' Achieves 89% LULC classification accuracy. |
| Formula | SPI = (Band 3 – Band 11) / (Band 3 + Band 11) — distinguishes solar panel reflectance from vegetation, bare soil, and water |
| Data source | Sentinel-2 Level 2A (10m optical) — free on Google Earth Engine |
| India coverage | Full India coverage, 5-day revisit period |
| Licence | CC BY-NC-ND 4.0 (paper) — Sentinel-2 data is free (ESA Copernicus open data) |

### Flaws of SPI

- Spectral confusion: Solar panels can be confused with metal rooftops, water bodies, and barren land — all have similar reflectance in SWIR. Requires post-processing with slope and land-cover masks.

- Monsoon limitation: Cloud cover during June–September blocks Sentinel-2 optical bands. Need to use cloud-free composites (median of dry-season months).

- No capacity estimation: SPI detects presence/absence of panels but cannot determine installed MW from imagery alone — needs panel area × efficiency × CF.

- 10m resolution limits: Individual rooftop panels are at the limit of Sentinel-2 resolution; small rooftop installations may be missed.

### Expected Outcomes After Integration

| Use Case | Method | Expected Outcome |
| --- | --- | --- |
| Fix Flaw 1 (district solar proxy) | Apply SPI to all TG + AP to get actual solar footprint per district. Compare to our pop-weighted proxy. | Ground-truth district solar MW without RTI — direct fix to biggest dataset limitation |
| Training set augmentation | Use SPI-detected plants as additional training samples (location known; estimate capacity from panel area) | Increase training set from 127 to 300+ plants |
| Validation of CEA data | Cross-check CEA's 4,360 MW TG against SPI-detected total area | Independent verification of our primary data source |

# Google Earth Engine SPI calculation

s2 = ee.ImageCollection('COPERNICUS/S2_SR').filterBounds(tg_boundary)

.filter(ee.Filter.lt('CLOUDY_PIXEL_PERCENTAGE', 10))

.filter(ee.Filter.calendarRange(11, 3, 'month'))  # dry season

.median()

SPI = s2.normalizedDifference(['B3', 'B11'])  # Band 3 = Green, Band 11 = SWIR

solar_mask = SPI.gt(0.15).And(slope.lt(5))   # threshold + slope filter

# PART 2 — COMPLETE FEATURE LIST

Below is the exhaustive list of every feature in the platform — existing ones already in the blueprint and new ones we're adding based on the integrations and flaws analysis. Features are grouped by module.

## 2.1  Module 1: Utility-Scale Site Analysis (Core)

### Existing Features (from Blueprint)

| Feature | Status | Description |
| --- | --- | --- |
| 42-Feature ML Suitability Score (0–1) | EXISTS | GHI, terrain, land use, infrastructure, climate, environmental, economic — ensemble RF+XGB+GB |
| Candidate Site Grid (5km TG, 10km national) | EXISTS | 30,000+ sites across India generated from GADM boundaries |
| Hard Exclusion Filters | EXISTS | Slope>5°, forest, water, urban, low GHI — reduces 10K TG sites to 2,856 viable |
| Capacity Estimation (GCR-based) | EXISTS | MW installable = usable area × GCR × panel_density |
| Generation Forecasting (Hybrid Physical+ML) | EXISTS | 70% physical baseline + 30% ML correction; 25-year lifetime with 0.5% degradation |
| Economic Analysis (LCOE, NPV, Payback) | EXISTS | CAPEX ₹4Cr/MW base + ₹50L/km grid connection; discount rate 8%; 25-year horizon |
| SHAP Explainability | EXISTS | Per-site feature importance breakdown — 'why is this site ranked high?' |
| Top 100 Sites Report (TG) | EXISTS | Ranked shortlist with full economic analysis and case study profiles |
| District Demand-Supply Gap Score | EXISTS | gap = (demand_mwh - solar_mwh) / demand_mwh — our original contribution |

### New Features (Adding Based on This Analysis)

| Feature | Status | Source / How |
| --- | --- | --- |
| pvlib Physics Engine (replaces custom formula) | NEW | pip install pvlib — replaces our PR formula with SAPM cell temp + Perez irradiance transposition |
| PVWatts V8 Benchmark Layer | NEW | NREL API — auto-run PVWatts for every site; show side-by-side with our forecast |
| GSA 250m GHI (replaces NASA POWER 55km) | NEW | GEE download — 220× resolution upgrade; enables per-district CF calculation |
| SPI Solar Panel Detection (fix Flaw 1) | NEW | Sentinel-2 + SPI formula — actual district solar footprint without RTI |
| AlphaEarth 64-dim Embeddings as ML Features | NEW | GEE GOOGLE/SATELLITE_EMBEDDING/V1/ANNUAL — additional 64 features per site |
| District-Specific Capacity Factor | NEW | NASA POWER district-level GHI → CF = (GHI/1000) × PR × (1-temp_loss) |
| Bifacial Panel Option | NEW | pvlib bifacial model — 5–10% more generation for appropriate sites |
| Single-Axis Tracker Option | NEW | pvlib tracker model — 15–25% more generation; adds civil work cost estimate |
| REopt Storage Optimisation | NEW | NREL REopt API — solar + BESS co-optimisation for top sites |
| EIA Category Screener | NEW | Auto-classify EIA A/B1/B2 based on capacity + protected area proximity |
| Floating Solar Identifier | NEW | ISRO water body layer + depth filter — flag suitable reservoirs/dams |
| Agrivoltaics Score | NEW | ISRO LULC agricultural + shade-tolerant crop database — dual-use sites |
| Climate Risk Composite Score | NEW | Cyclone + flood + hail + dust storm + extreme heat — per-site risk rating |

## 2.2  Module 2: Rooftop Solar Analysis

| Feature | Status | Description |
| --- | --- | --- |
| Building Footprint Detection (Google Open Buildings) | EXISTS | 300M+ India buildings; roof area estimation |
| Rooftop Suitability Score | EXISTS | Roof area, orientation, shading, structural type |
| PM Surya Ghar Subsidy Calculator | EXISTS | 1kW=₹30K, 2kW=₹60K, 3kW=₹78K (max) |
| DISCOM Tariff Database | EXISTS | Residential/commercial/industrial tariffs for top 10 states |
| Address-to-Rooftop Analysis | EXISTS | Geocode address → find building → compute capacity + payback |
| pvlib Rooftop Simulation | NEW | pvlib hourly simulation for residential — replaces our simple CF formula |
| 3D Shadow Modelling (DSM) | NEW | Copernicus DEM + building heights → hourly shading simulation |
| Net Metering Eligibility Check | NEW | State-wise NM limits, DISCOM policy status, application link |
| PM Surya Ghar Portal Integration | NEW | Live district data from pmsuryaghar.gov.in API |

## 2.3  Platform-Wide Features

| Feature | Status | Description |
| --- | --- | --- |
| Interactive Choropleth Map (district level) | EXISTS | Leaflet + React; districts coloured by gap score / suitability |
| Site Comparison Tool (up to 10 sites) | EXISTS | Side-by-side GHI, capacity, LCOE, accessibility radar chart |
| PDF Report Generator | EXISTS | Per-site downloadable report with all analysis outputs |
| CSV / KML Export | EXISTS | Site list export for developer GIS workflow |
| REST API (FastAPI) | EXISTS | POST /utility/analyze, GET /utility/site/{id}, POST /rooftop/analyze |
| Real-Time Data Updates (monthly) | NEW | Cron job: refresh NASA POWER + CEA data monthly; auto-retrain on 10+ new plants |
| RTI Status Tracker | NEW | Track pending RTI filings (TSREDCO, NREDCAP, DISCOMs) with due-date alerts |
| Multilingual Support (Hindi, Telugu) | NEW | i18next integration; priority for government/DISCOM users |
| Offline Mode (PWA) | NEW | Service worker caches top 100 TG sites for field inspection offline |
| Portfolio Optimiser | NEW | Select N sites to meet capacity target with geographic diversity + grid constraints |
| Hybrid Systems (solar + wind + storage) | NEW | REopt backend + wind data from NIWE; Year 1 roadmap |
| AlphaEarth Temporal Change Detector | NEW | Compare 2019 vs 2024 embeddings → flag land-use changed sites |

# PART 3 — REACT FRONTEND COMPLETE PLAN

## 3.1  Design Philosophy & Tech Stack

The frontend follows a 'dark data-dense dashboard' aesthetic — deep navy/slate backgrounds, amber/orange solar accents, monospace data displays, and smooth transitions. Think: Bloomberg Terminal meets Google Earth. Every user interaction reveals data, not marketing.

| Layer | Technology | Purpose | Version |
| --- | --- | --- | --- |
| Framework | React 18 | Core UI — function components, hooks, concurrent mode | 18.3 |
| Build Tool | Vite | Sub-second HMR, tree-shaking, ESM native | 5.x |
| Routing | React Router v6 | Client-side routing, nested routes, lazy loading | 6.x |
| Map | Leaflet + React-Leaflet | Interactive choropleth + site markers; lightweight (< React-Map-GL) | 4.x |
| Charts | Recharts | Suitability radar, LCOE bar, monthly generation line | 2.x |
| State Management | Zustand | Lightweight global state (selected sites, filters, user prefs) | 4.x |
| Data Fetching | TanStack Query (React Query) | Server state caching, background refetch, loading/error states | 5.x |
| Styling | Tailwind CSS + CSS Variables | Utility-first; CSS vars for theme tokens (dark/light mode) | 3.x |
| Animations | Framer Motion | Page transitions, map marker pop-ins, chart reveals | 11.x |
| Icons | Lucide React | Consistent icon set (already in project blueprint) | 0.383 |
| PDF Generation | React-PDF / @react-pdf/renderer | Client-side PDF reports from site analysis | 3.x |
| Forms | React Hook Form + Zod | Site filter form, rooftop address input, API key form | 7.x / 3.x |
| i18n | i18next + react-i18next | English / Hindi / Telugu | 14.x |

## 3.2  Application Routes & Page Map

| Route | Page / Component | Description |
| --- | --- | --- |
| / | HomePage | Hero with stats, two CTA buttons (Utility / Rooftop), live state counters |
| /utility | UtilityMapPage | Full-screen map with district overlay; site list sidebar; filter panel |
| /utility/site/:id | SiteDetailPage | Complete analysis for one candidate site — all charts, SHAP, PDF |
| /utility/compare | ComparisonPage | Side-by-side comparison of up to 10 selected sites |
| /utility/district/:name | DistrictPage | District profile: demand, solar, gap score, top sites within |
| /rooftop | RooftopPage | Address search → building finder → rooftop analysis result |
| /rooftop/result | RooftopResultPage | Full rooftop report: capacity, economics, subsidy, PDF download |
| /dashboard | AnalyticsDashboard | Admin/enterprise: usage stats, API call counts, custom portfolio |
| /portfolio | PortfolioOptimiser | Select target GW, constraints → find optimal site portfolio |
| /api-docs | APIDocs | Embedded Swagger/Scalar UI for the FastAPI backend |
| /about | AboutPage | Project, team, paper citation, open-source links |
| /methodology | MethodologyPage | Interactive explanation of dataset derivation + ML pipeline |

## 3.3  Key Components — Detailed Spec

### 3.3.1  SiteMap (Core Map Component)

The most important component. Full-screen Leaflet map with three layers:

| Layer | Type | Data Source | Colour Encoding |
| --- | --- | --- | --- |
| District Choropleth | GeoJSON polygon layer | GADM v4.1 district boundaries + our gap_score | Orange gradient: dark=high gap (needs solar), light=low gap |
| Candidate Sites | Circle markers (2,856 TG) | PostGIS candidate_sites table | Blue=low suitability → Green=high suitability; size=capacity MW |
| Existing Plants | Diamond markers | 127 training plants from CEA/MNRE | Yellow markers — helps users see where real plants are vs our candidates |

Key interactions: click district → show district info panel. Click site marker → open SiteCard popup. Filter sidebar toggles layer visibility.

### 3.3.2  FilterPanel (Left Sidebar)

| Filter | UI Control | Filters |
| --- | --- | --- |
| GHI Range | Dual-handle range slider | 1600–2400 kWh/m²/yr |
| Suitability Score | Range slider (0–1) | e.g., show only score > 0.7 |
| Capacity (MW) | Range slider (10–500 MW) | Min/max installable capacity |
| DISCOM | Multi-select checkboxes | TSSPDCL, TSNPDCL, APEPDCL, APCPDCL, APSPDCL |
| Land Use | Multi-select chips | Wasteland, Degraded, Agriculture (flagged) |
| Distance to Grid | Max slider (0–50 km) | Only show sites within X km of substation |
| LCOE Ceiling | Input (₹/kWh) | e.g., show sites where LCOE < ₹3.00 |
| Panel Type | Toggle: Standard / Bifacial / Tracker | Re-runs generation estimate with pvlib |

### 3.3.3  SiteDetailPage — Component Breakdown

| Section | Component | Data Shown |
| --- | --- | --- |
| Hero Banner | SiteHero | Site ID, location, coordinates, suitability score badge, DISCOM, district |
| Key Metrics Strip | MetricsRow | GHI ¦ Capacity MW ¦ Annual MWh ¦ LCOE ₹/kWh ¦ Payback years — 5 cards |
| Suitability Radar | RadarChart (Recharts) | 8-axis radar: solar resource, terrain, land use, infra, climate, env, grid, economic |
| SHAP Waterfall | SHAPChart (custom bars) | Top 10 features contributing to suitability — shows 'why this site scores X' |
| Monthly Generation | LineChart (Recharts) | 12-month AC output forecast; pvlib vs our hybrid; shaded monsoon period |
| 25-Year Projection | AreaChart (Recharts) | Annual generation declining 0.5%/yr; cumulative GWh secondary axis |
| Economic Summary | EconomicsTable | CAPEX, OPEX/yr, Revenue/yr, NPV, LCOE, Payback — with sensitivity toggle |
| Location Intelligence | LocationCards | Distance to nearest substation, road, city, protected area, water body |
| Comparison Trigger | CompareButton | Add this site to comparison basket (max 10) |
| Export Section | ExportPanel | Download PDF report / CSV row / KML point / JSON API response |

### 3.3.4  RooftopAnalysisFlow — Step-by-Step UX

The rooftop module uses a guided 3-step wizard:

| Step | Component | Input | Output |
| --- | --- | --- | --- |
| Step 1: Location | AddressInput | Type address or drop pin on map | Geocoded lat/lon + nearest building polygon from Google Open Buildings |
| Step 2: System Config | SystemConfigForm | Ownership type (owned/rented), monthly bill amount, panel preference | System size recommendation + subsidy eligibility check |
| Step 3: Results | RooftopResultDashboard | None (auto-computed) | Capacity kW, annual kWh, system cost, subsidy, net cost, payback years, monthly savings |

### 3.3.5  DistrictPage — District Profile

| Section | Description |
| --- | --- |
| Demand vs Supply Bar | Horizontal bar: demand_mwh vs current_solar_mwh — visual gap instantly clear |
| Gap Score Gauge | Circular gauge 0–100%; red=critical gap, green=surplus |
| Top 5 Sites in District | Mini site cards linking to SiteDetailPage |
| District Stats Grid | Population, Census year, DISCOM, avg GHI, area km², land use breakdown |
| Data Provenance | Source badges: TGERC/APERC/CEA icons with year — shows data is authoritative |
| RTI Status Badge | If district solar data is pop-weighted proxy: yellow 'ESTIMATED' badge + RTI filing status |

## 3.4  State Management — Zustand Stores

// stores/siteStore.js

const useSiteStore = create((set) => ({

selectedSites: [],        // comparison basket (max 10)

hoveredSiteId: null,      // map hover highlight

filters: { ghiMin: 1700, ghiMax: 2400, scoreMin: 0, ... },

addToComparison: (site) => set(s => ({

selectedSites: [...s.selectedSites.slice(0,9), site] })),

setFilter: (key, val) => set(s => ({

filters: {...s.filters, [key]: val} })),

}))

// stores/mapStore.js

const useMapStore = create((set) => ({

activeLayer: 'suitability',  // 'suitability' | 'gap_score' | 'ghi' | 'lcoe'

visibleDISCOMs: ['TSSPDCL','TSNPDCL','APEPDCL','APCPDCL','APSPDCL'],

showExistingPlants: true,

showCandidateSites: true,

}))

## 3.5  API Integration (TanStack Query Hooks)

// hooks/useSites.js

export const useSites = (filters) => useQuery({

queryKey: ['sites', filters],

queryFn: () => api.get('/utility/sites', { params: filters }),

staleTime: 5 * 60 * 1000,  // 5 min cache

})

// hooks/useSiteDetail.js

export const useSiteDetail = (id) => useQuery({

queryKey: ['site', id],

queryFn: () => api.get(`/utility/site/${id}`),

staleTime: 30 * 60 * 1000,  // 30 min — site data doesn't change often

})

// hooks/useRooftop.js

export const useRooftopAnalysis = () => useMutation({

mutationFn: (payload) => api.post('/rooftop/analyze', payload),

})

## 3.6  Folder Structure

frontend/

├── src/

│   ├── pages/

│   │   ├── HomePage.jsx

│   │   ├── UtilityMapPage.jsx

│   │   ├── SiteDetailPage.jsx

│   │   ├── ComparisonPage.jsx

│   │   ├── DistrictPage.jsx

│   │   ├── RooftopPage.jsx

│   │   └── PortfolioOptimiser.jsx

│   ├── components/

│   │   ├── map/

│   │   │   ├── SiteMap.jsx          # Main Leaflet map

│   │   │   ├── DistrictLayer.jsx    # Choropleth GeoJSON layer

│   │   │   ├── SiteMarkers.jsx      # Candidate site markers

│   │   │   └── MapControls.jsx      # Layer toggle UI

│   │   ├── charts/

│   │   │   ├── SuitabilityRadar.jsx

│   │   │   ├── MonthlyGenLine.jsx

│   │   │   ├── LifetimeArea.jsx

│   │   │   └── SHAPWaterfall.jsx

│   │   ├── site/

│   │   │   ├── SiteCard.jsx

│   │   │   ├── SiteHero.jsx

│   │   │   ├── MetricsRow.jsx

│   │   │   └── ExportPanel.jsx

│   │   ├── rooftop/

│   │   │   ├── AddressInput.jsx

│   │   │   ├── SystemConfigForm.jsx

│   │   │   └── RooftopResultDashboard.jsx

│   │   └── ui/                     # Shared: Button, Badge, Tooltip, Modal

│   ├── stores/

│   │   ├── siteStore.js

│   │   ├── mapStore.js

│   │   └── userStore.js

│   ├── hooks/

│   │   ├── useSites.js

│   │   ├── useSiteDetail.js

│   │   └── useRooftop.js

│   ├── services/

│   │   └── api.js                  # Axios instance + interceptors

│   ├── i18n/

│   │   ├── en.json

│   │   ├── hi.json

│   │   └── te.json

│   └── App.jsx

## 3.7  Performance Plan

| Challenge | Solution | Target |
| --- | --- | --- |
| 30,000 site markers on map | Cluster markers with react-leaflet-cluster; only show top 500 in current viewport via PostGIS ST_Within query | < 2s initial map load |
| District GeoJSON (61 polygons) | Simplify polygons to 1% tolerance (turf.js simplify); serve as pre-built GeoJSON static file | < 100ms choropleth render |
| Site detail API call | Redis cache on FastAPI; staleTime=30min in React Query | < 200ms cached, < 2s fresh |
| Rooftop analysis (slow) | Show progress steps (geocoding → building match → pvlib sim → economics); stream results | User sees partial results in < 3s |
| PDF generation | Generate client-side with @react-pdf/renderer (no server round-trip) | < 5s PDF ready |
| Bundle size | Code-split each page with React.lazy(); Leaflet (~140kb) loaded separately | < 250kb initial bundle |

## 3.8  Deployment Configuration

| Component | Service | Cost | Notes |
| --- | --- | --- | --- |
| React Frontend | Vercel (free tier) | ₹0 | Auto-deploy from GitHub main branch; CDN edge caching globally |
| FastAPI Backend | AWS EC2 t3.small OR Railway.app | ~₹1,500/month OR free tier | Docker container; auto-scale on traffic spikes |
| PostgreSQL + PostGIS | AWS RDS db.t3.micro OR Supabase | ~₹2,000/month OR free 500MB | PostGIS essential for spatial queries; Supabase free tier for dev |
| Redis Cache | AWS ElastiCache t3.micro OR Upstash | ~₹1,000/month OR free tier | Cache API responses; <2s response target |
| File Storage (PDFs, models) | AWS S3 | ~₹200/month | Store ML model .pkl, pre-generated PDFs, GeoJSON files |
| Domain | Namecheap / GoDaddy | ~₹1,000/year | solarsite-india.org |
| Total Estimated Monthly | Development/launch phase | ~₹4,700/month | Upgrade as users grow; free tiers sufficient for first 6 months |

# NOTE ON MEMORY — PROJECT 'SOLARSITE' SAVED

All critical SolarSite-India project context has been saved to Claude's memory under the label 'SolarSite'. In any future conversation, Claude will have access to:

- Project status (Stages 0–2 done, Stage 3 ML in progress), team size, tech stack, target journal

- All authoritative dataset values (TG demand, AP demand, CEA solar figures, Census populations)

- The 8 known flaws and their fixes

- ML architecture (42 features, ensemble weights, training label source, AlphaEarth integration)

- All documents produced so far (v2 Excel, complete documentation, methodology doc, team briefing)

- Integration tools (pvlib, PVWatts V8, REopt, GSA, AlphaEarth, SPI) and business model

⚠️ Memory Note: Claude's memory persists across conversations but not across different accounts or incognito mode. Each team member should share project context if using Claude on their own account.

✅ How to recall: In any new conversation just say 'SolarSite project' and Claude will immediately have full context without needing re-explanation.

End of Document